RSS Component Classification and Numbering Guidelines

Below are the guidelines for beamline component classifications and numbering schemes which are to be used in the Beamline Component Database.

F	Front End Components

G	Guillotines

I	Interlocks

K	Collimators

L	Labyrinths

M	Masks

P	Shutters and Stops

S	Supplemental Shielding

T	Transport

X	Extra elements (which do not fit the categories above)

Y	Survey Ports

#	Enclosure

For standardization purposes, whenever any beam is stopped completely,

the component will be a "P" designation.

The Beamline Component Database SEQUENCE NUMBERS are ordered by the

component classifications.  If you have 12 configuration control components in one hutch, the sequence numbers are not 1 through 12. Rather, a listing should look like:

1-BM-A-#	Enclosure Shielding

1-BM-A-F-01	Front End Components

1-BM-A-G-01	Guillotines

1-BM-A-L-01	Labyrinths

1-BM-A-Y-01	Survey Ports

1-BM-A-K-01	K5-20 Collimator

1-BM-A-M-01	M9-30 Mask

1-BM-A-K-02	Bremsstrahlung Collimator

1-BM-A-P-01	Beam Stop

1-BM-A-K-03	Bremsstrahlung Collimator

1-BM-A-P-02	P4-20 Integral Shutter

1-BM-A-T-01	Shielded Beam Transport

1-BM-A-T-02	Flange Box(es), Mailboxes

In the database, labyrinths will be listed as one element.  However, when making RSS tags, each labyrinth will have a distinct number.  If a labyrinth has an inside and an outside lid, two tags with the same number will have to be created.

1-BM-A-L-01	Labyrinth #01

1-BM-A-L-02	Labyrinth #02

1-BM-A-L-03	Labyrinth #03 (inside)

1-BM-A-L-03	Labyrinth #03 (outside)

When RSS tagging labyrinths, the labyrinths are numbered sequentially, starting with the upstream-most roof labyrinth. When a new labyrinth is added somewhere in the middle of the sequence, the higher number tags are shifted to maintain the sequence.